fe

By Connor Ingle

Layer 2 Attacks Lab

Cisco Certified Networking Program

Purpose:

The purpose of this lab was to configure and show evidence of three different layer 2 attacks on a small network

Background:

ARP is a protocol that allows network communications to reach a specific device on the network. ARP translates IP addresses to a MAC address and vice versa. If the host doesn’t know the MAC address for a certain IP address, it sends out an ARP request packet. The protocol was not designed for security, so it does not verify that the response to the ARP reply came from an authorized device. This leaves a place for ARP spoofing attacks to occur. ARP spoofing allows the attacker to see communication between network devices. For an ARP spoofing attack, the attacker must be connected to the network and know the IP addresses of at least two different devices. Then they use a spoofing tool to send out fake ARP responses. The responses say that the correct MAC address for both the IP addresses is the attacker’s MAC address, allowing them to connect to both devices, instead of each other. Now the attacker can see the packets meant for the other devices and is secretly in the middle of the communications.

VLAN hopping is where an attacker gains unauthorized access to a VLAN. They can then break into other VLANs in the same network. VLAN hopping allows attackers to bypass the segmented network traffic, which is used by VLANs to improve security and performance. This allows attackers to gain access to sensitive data that is supposed to be isolated and secure. Once the network segmentation is compromised, the risk of widespread data breaches escalates. Attackers can also use VLAN hopping to overload networks which leads to the integrity of the network being compromised as well.

A MAC flooding attack is when an attacker sends many packets with fake or random MAC addresses, which quickly fills up the targets MAC address table. The table will eventually overflow, which means the switch can no longer forward traffic directly to the intended device. The switch will instead broadcast all incoming traffic to every port, which makes all the data visible to any connected devices. An administrator can detect spikes in MAC addresses using tools like Wireshark or just monitoring rapid MAC address changes directly from the MAC address table. You can prevent MAC flooding attacks by enabling port security which sets a certain number of MAC addresses to be allowed on each port preventing the MAC address table from being overwhelmed. You could also separate your network into VLANs so that the impact of the MAC flooding attack is limited to the VLAN that the attacker is trying to overflow.

Lab Summary:

In this lab we created a small network and performed different layer 2 attacks

Lab Commands:

Network Diagram:

Configs:

Courier font no space

Show Commands:

Problems:

One problem that we had was figuring out how to do the ARP poisoning attack, because we started making a network and followed some instructions, but we didn’t know that we had to have a virtual machine to run the ARP command to initiate the attack. We then found a free virtual machine and set it up so that we could use the attack, but you also had to download the special ARP spoof command to use in the virtual machine. So, after we did that and set up a small network for the attack, we still had problems with connectivity.

Conclusion:

In conclusion, it is important to know what to look for and how to spot a layer 2 attack and how to mitigate them. Layer 2 is a fundamental layer, more specifically the Data Link layer because it provides the network infrastructure. It is also important because many security protocols focus on higher layers and layer 2 can be exploited in ARP poisoning, MAC flooding, or VLAN hopping.